# Apollo AL-924/AF-924

## Compact 4 Line Alphanumeric Pagers

| Features: - Compact pager with 4 line display for 84 Alpha characters. - Total capacity: 8150 characters memory for POCSAG. - Button programmable.. - 6 Capcode capacity which can be personal or maildrop with no frame restrictions. - Time, date display - International date format dd-mm-yy - 16 sets of memory, eight lockable - Four alert modes: - Beep - Vibrator - Melody - Sleep - Ten melodies, four beeper sounds - LCD backlight for easy viewing - Alarm function - Low Battery indicator and alarm - Suitable for Network or Local Area Paging. - 12 Month service warranty | A compact hand programmable 4 line alphanumeric pager with zoom function (zooms display to 2 lines large characters) and synthesised receiver (pocsag). Specifications - Protocols Pocsag (AL924) Flex (AF924) - Frequencies: 132–174 MHz 443–473 MHz - Frequency Set Synthesised (AL924) Crystalled (AF924) - Baud rate: 512, 1200 or 2400bps - Sensitivity 512 BPS--5uV/M 1200 BPS--7uV/M - Spurious Rejection 40db below carrier - Channel Spacing 25 kHz - Selectivity 60db @ 25 kHz - Alert Tone Loudness 85db @ 30 cm - Frequency deviation +4.5 kHz - Cap codes 6 Cap Codes (AL924) - Dimensions 61 x 42 x 18 mm - Battery One 1.5 V AAA Alkaline AL901/AF901 Numeric Pager Top display synthesised (AL901) numeric pager available in VHF and UHF. Button programmable. |
| --- | --- |