VHF, UHF, 900 MHz, Pocsag & FLEX

- 84 Alphanumeric One Page Display - Blue Back Light Display - 16 Memory Slots - Total Capacity: 28,672 Letters for POCSAG, 32,000 Letters for FLEX - 24,576 (POCSAG), 27,000(FLEX) Character for Mail Drop - 16 Memory Slots, 10 Sets Full Message - 4 Beep Alerts, 10 Melody Alerts, Vibration Sleep Mode - Message Full Indicator - 5 Daily Alarm Sets - 8 Message Locks - Extra Long Battery Life; - Mail Drop Alert - Real Time Clock Calendar - Extensive Compatibility - 10 Personal Extra Personal Slots - Duplicate Message Indicator - Unread Message Display - Message Time Stamp - Month, Date, Year, Time Display - Selective Message Lock/Delete - Lithium Battery Backup - Reminder Alert / Zoom Feature - Selective PC or Hand Programming - Up to 6 Capcodes Capability for POCSAG, 16 Capcodes Capability for FLEX - AutoSleep, Auto Scroll. * FLEX Model Only.

Specifications

Frequency; 138~174 MHz, 408~473 MHz and, 929~932 MHz. Paging Sensitivity; 512 BPS--5uV/M, 1200 BPS--7uV/M, 2400bps--9uV/M. Spurious Rejection; 40db below carrier. Channel Spacing; 25 kHz. Selectivity; 60db @ 25 kHz. Code Format; FLEX(AF-A25), Pocsag(AL-A25). Bit Rate; 512 / 1200 / 2400 BPS for POCSAG, 1600 / 3200 / 6400 BPS for FLEX. Alert Tone Loudness; 85db @ 30 cm (12 inches). Frequency deviation; +4.5 kHz. Cap codes; 6 Cap Codes for POCSAG, 16 Cap Codes for FLEX. Display; 84 Letters. Dimensions; 3.1 x 2.1 x 0.7 inches. Weight (with battery); Approximately 0.23lb. Battery; One 1.5 V AA Alkaline Battery.