PDR2000 Specifications

General

Case Construction	Steel

Physical Dimensions	220mm x 72mm x 33mm

Operating Voltage	9 – 15V DC

Nominal Voltage	12V

Power Usage (Maximum)	1W (ie, 85mA @ 12V)

Real Time Clock	Battery backed Y2K compliant real time clock

Processor

Processor Speed	16MHz

Non Volatile RAM	256Kb

Volatile RAM	20Kb

Flash memory	256Kb (64Kb set aside for static configuration data)

Input and Output

Parallel Port	Centronix

Serial Port	RS232

Relay	1A @ 12V

Receiver

Type	Synthesised super heterodyne with external antenna

Frequency	168.950 MHz

Protocol	Pocsag

Channel Spacing	25 KHz

Signal Format	POCSAG

Data Transmission Rate	512/1200/2400 bps (POCSAG) ,

Modulation  System	FSK

Frequency Deviation	 +/- 4,5 KHz

Receiving Sensitivity	5 uV at 1200 bps

Selectivity	better than 60 dB

Image Reject	ion	better than 55 dB

Spurious Rejection	better than 55 dB

Frequency Stability	10 ppm at -10C~+60C ,