RCM2000 Relay Board Control

The Relay Control Board provides 32 relays which may be controlled from a PDT2000/ PDR2000 or directly from a PC or other serial device.  Multiple relay boards may be daisy chained up to 2056 relays

There are three methods of control:

1. Capcode	Any one relay can be programmed to activate on any message received on a particular capcode.  Note: multiple instances of the same capcode can be setup to control multiple relays.

2. Message text 	Any one relay can be programmed to activate on any message received containing specified Text.  Note: multiple instances of the same Text can be setup to control multiple relays.

3. Direct protocol 	A standard message protocol that directly controls the relay board  without any additional programming setup.  This protocol, described below, enables relays to be activated via specific text contained in alphanumeric messages without presetting in PDT.

Direct Protocol Commands:

There are 4 commands to control the relay board:

1. Set relay x	Sxx%

2. Clear relay x	Cxx%

3. Timed set relay x	Txx,yyyy%

4. Clear all relays	A%

Where xx is the relay number. When x is between 1 and 32 it corresponds to a local relay, when it's 33 or above it corresponds to a relay further down the chain; (for relay numbers higher than 32 the relay control board subtracts 32 from the relay number and sends the command on to the next RCB.)  yyyy is time in msecs.

For example:

S12%			Set relay 12

C01%			Clear relay 1

T5,4000%		Timed set of relay 5 for 4000ms, i.e. 4s

A%			Clear all relays

Programming the PDT2000/PDR2000

There are 2 serial port output settings for the PDT as follows:

Output to Relay Board is used for methods 1 & 2 above and Direct To Relay Board is used for method 3.

If methods 1 or 2 is used then the other settings must be set in Capcodes and Message Relay Options.  Method 3 only requires that the output to serial is selected for the capcode.

Serial port settings:  57600,8,n,1

1	8	1	8

J3 	J2

Connections:

A. PDT Com2 Output to Relay Board: RJ11 - RJ45:

PDT	Relay Board

RJ10(Serial 2)	RJ45(J2 chain)

Pin1(RxD0) --------------------------------------------------------- Pin2(TxD0)

Pin2(TxD0) --------------------------------------------------------- Pin3(RxD0)

Pin4(Ground) ------------------------------------------------------- Pin8(Ground)

B. Daisy Chaining from one Relay Board to Another: RJ45 – RJ45

First Relay Board	Second Relay Board

RJ45(J3 prgm)	RJ45(J2 chain)

Pin2(RxD1) --------------------------------------------------------- Pin2(TxD0)

Pin3(TxD1) --------------------------------------------------------- Pin3(RxD0)

Pin8(Ground) ------------------------------------------------------- Pin8(Ground)

C. PC Com Port to Relay Board: DB9(F) – RJ45

PC	Relay Board

DB9(Female)	RJ45(J2 chain)

Pin2(Rx) ------------------------------------------------------------- Pin2(TxD0)

Pin3(Tx) ------------------------------------------------------------- Pin3(RxD0)

Pin5(Ground) ------------------------------------------------------- Pin8(Ground)

Specifications

| No. Relay Outputs per board | 32 |
| --- | --- |
| Relay Load Capacity | 250VAC/30VDC 3A |
| Serial Ports | 2 (command input and daisy chain output) |
| Relay Connectors | Screw terminals rated 300V/20A |
| Serial Connectors | RJ45 (3 pins used) |
| Power | 12Vdc |
| Current | 50-100mA + 16-20mA per relay activated, |
|  | Max 800mA |
| Dimensions | 194x147x35mm (approx7.5”x6”x1.25”) |