iPage Personal Edition Installation and Setup Instructions For Dial-Up TAP

Installing the iPage software

- Load the iPage CDROM into the PC on which it is run.  This PC will need to be connected to a modem or to the SitePage Paging Unit.

- Run iPage2PESetup.exe

- Run iPage.  A pager icon will appear on the taskbar indicating that the iPage server has loaded.

- Right click on the icon and select Stop

- Right click on the icon again and select “Edit Devices”

- Add a device as follows:

Device Name: 	Direct TAP (eg)

Device Type:	Direct TAP

ComPort:	1 	(Change if the TAP device is connected to a different COM Port)

Hit the Test Device button to ensure the device has been detected then hit OK.

- Start the iPage Server by right clicking on the Taskbar icon and selecting Start.

- You may also wish to change the start up settings by right clicking on the Taskbar icon and selecting Options.

Setting up the iPage Personal Edition Software

- The iPage Personal Edition main window should be visible on your desktop.

- Next setup a Carrier as follows:

Settings|Carriers|Add

Carrier Name:	{Direct TAP Device Name}

Carrier Type: 	TAP Direct Access

Server: 	iPage Server on Local Host

Max Msge Length:	200	(Some devices may have a different setting)

Modem baud rate:	9600	(This may need to be changed according to the parameters of the TAP device)

Data Settings:	7,E,1	(This is the most common setting however some devices may use 8,N,1)

- You can now setup Contacts as follows:

Settings|Contacts|Add

Name:	“Name or description of the person carrying the pager”

Carrier:	{Direct TAP Device Name}

Pager ID:	(Generally  this will be the capcode of the pager)

- Add Contacts to your database by left clicking on the Database Name and then Right clicking again and selecting Add Contact.  Select all you wish to add using Ctrl Click and then hit OK.

iPage is now ready to send messages.