iPage Personal Edition Installation and Setup Instructions

Installing the iPage software

- Load the iPage CDROM into the PC on which it is run.  This PC will need to be connected to a modem or to the SitePage Paging Unit.

- Run iPage2PESetup.exe

- Run iPage.  A pager icon will appear on the taskbar indicating that the iPage server has loaded.

- Right click on the icon and select Stop

- Right click on the icon again and select “Edit Devices”

- Add a device as follows:

Device Name: 	Local Modem

Device Type:	Modem

ComPort:	1 	(Change if the modem is connected to a different COM Port)

Hit the Test Device button to ensure the modem has been detected then hit OK.

7. Start the iPage Server by right clicking on the Taskbar icon and selecting Start.

Setting up the iPage Personal Edition Software

- The iPage Personal Edition main window should be visible on your desktop.

- Next setup a Carrier as follows:

Settings|Carriers|Add

Carrier Name:	Telecom

Carrier Type: 	TAP Modem Access

Server: 	iPage Server on Local Host

Phone:	026 4001283	(If on a PABX extension add the number required to get an outside line e.g. 1,)

Max Msge Length:	200

Modem baud rate:	9600	(If communications are inconsistent try a low speed)

Data Settings:	7,E,1

- You can now setup Contacts as follows:

Settings|Contacts|Add

Name:	“Name or description of the person carrying the pager”

Carrier:	Telecom

Pager ID:	The 7 digit dial number provided by Telecom

- Add Contacts to your database by left clicking on the Database Name and then Right clicking again and selecting Add Contact.  Select all you wish to add using Ctrl Click and then hit OK.

iPage is now ready to send messages.

56K Modems	High speed modems occasionally have difficulty working with the TAP protocol used by paging programmes.  If you have difficulty connecting to the paging company try editing the modem Init string (Server|Edit Devices) to the following:   AT&fQ0V1s32=)x2I3